| Clause number and objective | Standard | Complies / Does not comply / Variation required Choose relevant option and provide comments. Delete others. Does it meet the objective? Is a variation of the standard required? Is further information required? |
| --- | --- | --- |
| Clause 54.02-1 Neighbourhood character objective To ensure that the design respects the existing neighbourhood character or contributes to a preferred neighbourhood character. To ensure that the design responds to the features of the site and the surrounding area. | Standard A1 The design response must be appropriate to the neighbourhood and the site. The proposed design must respect the existing or preferred neighbourhood character and respond to the features of the site. | N/A  Complies  Does not comply Variation required Is there an obvious character to the neighbourhood? What elements contribute to the existing/preferred neighbourhood character? (Comment on the height, bulk, front and side setbacks of your proposal. Is the site in a substantial change area, incremental with access area, limited change area or minimal change area – refer to Clause 16.01-1L of the Latrobe Planning Scheme). |
| Clause 54.02-2 Integration with the street objectives To integrate the layout of development with the street. | Standard A2 Dwellings should be oriented to front existing and proposed streets. High fencing in front of dwellings should be avoided if practicable. Dwellings should be designed to promote the observation of abutting streets and any abutting public open spaces. | N/A  Complies  Does not comply Variation required How is the dwelling designed to integrate with the street? (Comment on the accessibility for pedestrians and vehicles) Is there high front fencing proposed for this development? |
| Clause 54.03-1 Street setback objective To ensure that the setbacks of buildings from a street respect the existing or preferred neighbourhood character and make efficient use of the site. | Standard A3 Walls of buildings should be set back from streets the distance specified in Table A1. Porches, pergolas and verandahs that are less than 3.6 metres high and eaves may encroach not more than 2.5 metres into the setbacks of this standard. Residential Growth Zone 1, 3 & 4: 3 metres Porches, pergolas and verandah that are less than 3.6 metres high and eaves may encroach not more than 2.5 metres into the setback of this standard Neighbourhood Residential Zone 2 & 3 7.5 metres Porches, pergolas and verandahs that are less than 3.6 metres high and eaves may encroach not more than 2.5 metres into the setbacks of this standard. | N/A  Complies  Does not comply Variation required How does the proposal meet with this standard? Does the front porch/pergola or verandah encroach within the front setback? If a variation to the standard is sought, adequate justification is required. Describe the existing setbacks within the street. An existing site plan should be submitted showing the adjoining and directly opposite properties within the street and their front setbacks. |
| Clause 54.03-2 Building height objectives To ensure that the height of buildings respects the existing or preferred neighbourhood character. | Standard A4 The maximum building height should not exceed the maximum height specified in the zone, schedule to the zone or an overlay that applies to the land. If no maximum height is specified in the zone, schedule to the zone or an overlay, the maximum building height should not exceed 9 metres, unless the slope of the natural ground level at any cross section wider than 8 metres of the site of the building is 2.5 degrees or more, in which case the maximum building height should not exceed 10 metres. Changes of building height between existing buildings and new buildings should be graduated. | N/A  Complies  Does not comply Variation required Is the dwelling height graduated amongst existing development? |
| Clause 54.03-3 Site coverage objective To ensure that the site coverage respects the existing or preferred neighbourhood character and responds to the features of the site. | Standard A5 The site area covered by buildings should not exceed 60 percent. Neighbourhood Residential Zone 2 & 3 30 percent Neighbourhood Residential Zone 4 50 percent | N/A  Complies  Does not comply Variation required What is the site coverage? |
| Clause 54.03-4 Permeability objectives To reduce the impact of increased stormwater run-off on the drainage system. To facilitate on-site stormwater infiltration. | Standard A6 At least 20 percent of the site should not be covered by impervious surfaces. Neighbourhood Residential Zone 2 & 3 50 percent | N/A  Complies  Does not comply Variation required What is the permeability of the site? |
| Clause 54.03-5 Energy efficiency objectives To achieve and protect energy efficient dwellings. To ensure the orientation and layout of development reduce fossil fuel energy use and make appropriate use of daylight and solar energy. | Standard A7 Buildings should be: - Oriented to make appropriate use of solar energy. - Sited and designed to ensure that the energy efficiency of existing dwellings on adjoining lots is not unreasonably reduced. - Sited and designed to ensure that the performance of existing rooftop solar energy systems on dwellings on adjoining lots in a General Residential Zone, Neighbourhood Residential Zone or Township Zone are not unreasonably reduced. The existing rooftop solar energy system must exist at the date the application is lodged. Living areas and private open space should be located on the north side of the development, if practicable. Dwelling should be designed so that solar access to north-facing windows is maximised. | N/A  Complies  Does not comply Variation required How is the dwelling designed to take advantage of solar energy? (Comment on location of living rooms and private open space areas). Is the dwelling designed so that solar access to north facing windows is maximised? |
| Clause 54.03-6 Significant trees objectives To encourage development that respects the landscape character of the neighbourhood. To encourage the retention of significant trees on the site. | Standard A8 Development should provide for the retention or planting of trees, where these are part of the neighbourhood character. Development should provide for the replacement of any significant trees that have been removed in the 12 months prior to the application being made. | N/A  Complies  Does not comply Variation required Have significant trees been removed from the site within the past 12 months? |
| Clause 54.04-1 Side and rear setbacks objective To ensure that the height and setback of a building from a boundary respects the existing or preferred neighbourhood character and limits the impact on the amenity of existing dwellings. | Standard A10 A new building not on or within 200mm of a boundary should be set back from side or rear boundaries: - At least the distance specified in a schedule to the zone, or - If no distance is specified in a schedule to the zone, 1 metre, plus 0.3 metres for every metre of height over 3.6 metres up to 6.9 metres, plus 1 metre for every metre of height over 6.9 metres. Sunblinds, verandahs, porches, eaves, fascias, gutters, masonry chimneys, flues, pipes, domestic fuel or water tanks, and heating or cooling equipment or other services may encroach not more than 0.5 metres into the setbacks of this standard. Landings having an area of not more than 2 square metres and less than 1 metre high, stairways, ramps, pergolas, shade sails and carports may encroach into the setbacks of this standard. General Residential Zone 1 & 3 A new building not on or within 200mm of one side boundary should be set back from that side boundary at least 1.5 metres plus 0.3 metres for every metre of height over 3.6 metres up to 6.9 metres, plus 1 metre for every metre of height over 6.9 metres Neighbourhood Residential Zone 2 A new building not on or within 200mm of a side boundary should be set back from that side boundary at least 2 metres plus 0.3 metres for every metre of height over 3.6 metres up to 6.9 metres, plus 1 metre for every metre of height over 6.9 metres Neighbourhood Residential Zone 3 A new building not on or within 200mm of a side boundary should be set back from that side boundary at least 3 metres plus 0.3 metres for every metre of height over 3.6 metres up to 6.9 metres, plus 1 metre for every metre of height over 6.9 metres Neighbourhood Residential Zone 4 A new building not on or within 200mm of a side boundary should be set back from that side boundary at least 1.5 metres plus 0.3 metres for every metre of height over 3.6 metres up to 6.9 metres, plus 1 metre for every metre of height over 6.9 metres | N/A  Complies  Does not comply Variation required Do the proposed setbacks comply with the formula contained in the standard? |
| Clause 54.04-2 Walls on boundaries objective To ensure that the location, length and height of a wall on a boundary respects the existing or preferred neighbourhood character and limits the impact on the amenity of existing dwellings. | Standard A11 A new wall constructed on or within 200mm of a side or rear boundary of a lot or a carport constructed on or within 1 metre of a side or rear boundary of lot should not abut the boundary: - For a length of more than the distance specified in a schedule to the zone; or - If no distance is specified in a schedule to the zone, for a length of more than: - 10 metres plus 25 per cent of the remaining length of the boundary of an adjoining lot, or - Where there are existing or simultaneously constructed walls or carports abutting the boundary on an abutting lot, the length of the existing or simultaneously constructed walls or carports, whichever is the greater. A new wall or carport may fully abut a side or rear boundary where slope and retaining walls or fences would result in the effective height of the wall or carport being less than 2 metres on the abutting property boundary. A building on a boundary includes a building set back up to 200mm from a boundary. | N/A  Complies  Does not comply Variation required Describe the wall heights and lengths and how they comply with the standard. |
| Clause 54.04-3 Daylight to windows objective To allow adequate daylight into existing habitable room windows. | Standard A12 Buildings opposite an existing habitable room window should provide for a light court to the existing window that has a minimum area of 3 square metres and minimum dimension of 1 metre clear to the sky. The calculation of the area may include land on the abutting lot. Walls or carports more than 3 metres in height opposite an existing habitable room window should be set back from the window at least 50 per cent of the height of the new wall if the wall is within a 55 degree arc from the centre of the existing window. The arc may be swung to within 35 degrees of the plane of the wall containing the existing window. Where the existing window is above ground floor level, the wall height is measured from the floor level of the room containing the window. | N/A  Complies  Does not comply Variation required Have the required light courts been achieved? Do all windows have access to the required level of daylight? |
| Clause 54.04-4 North facing windows objective To allow adequate solar access to existing north-facing habitable room windows. | Standard A13 If a north-facing habitable room window of an existing dwelling is within 3 metres of a boundary on an abutting lot, a building should be setback from the boundary 1 metre, plus 0.6 metres for every metre of height over 3.6 metres up to 6.9 metres, plus 1 metre for every metre of height over 6.9 metres, for a distance of 3 metres from the edge of each side of the window. A north-facing window is a window with an axis perpendicular to its surface oriented north 20 degrees west to north 30 degrees east. | N/A  Complies  Does not comply Variation required Are there north facing windows on abutting lots which require consideration? If so, describe how the standard is met. |
| Clause 54.04-5 Overshadowing open space objective To ensure buildings do not unreasonably overshadow existing secluded private open space. | Standard A14 Where sunlight to the secluded private open space of an existing dwelling is reduced, at least 75 per cent, or 40 square metres with minimum dimension of 3 metres, whichever is the lesser area, of the secluded private open space should receive a minimum of five hours of sunlight between 9 am and 3 pm on 22 September. If existing sunlight to the secluded private open space of an existing dwelling is less than the requirements of this standard, the amount of sunlight should not be further reduced. | N/A  Complies  Does not comply Variation required Describe how the level of overshadowing associated with the dwelling is within the requirements of the standard. |
| Clause 54.04-6 Overlooking objective To limit views into existing secluded private open space and habitable room windows. | Standard A15 A habitable room window, balcony, terrace, deck or patio should be located and designed to avoid direct views into the secluded private open space of an existing dwelling within a horizontal distance of 9 metres (measured at ground level) of the window, balcony, terrace, deck or patio. Views should be measured within a 45 degree angle from the plane of the window or perimeter of the balcony, terrace, deck or patio, and from a height of 1.7 metres above floor level. A habitable room window, balcony, terrace, deck or patio with a direct view into a habitable room window of existing dwelling within a horizontal distance of 9 metres (measured at ground level) of the window, balcony, terrace, deck or patio should be either: - Offset a minimum of 1.5 metres from the edge of one window to the edge of the other. - Have sill heights of at least 1.7 metres above floor level. - Have fixed, obscure glazing in any part of the window below 1.7 metre above floor - level. - Have permanently fixed external screens to at least 1.7 metres above floor level and be no more than 25 per cent transparent. Obscure glazing in any part of the window below 1.7 metres above floor level may be openable provided that there are no direct views as specified in this standard. Screens used to obscure a view should be: - Perforated panels or trellis with a maximum of 25 per cent openings or solid translucent panels. - Permanent, fixed and durable. - Designed and coloured to blend in with the development. This standard does not apply to a new habitable room window, balcony, terrace, deck or patio which faces a property boundary where there is a visual barrier at least 1.8 metres high and the floor level of the habitable room, balcony, terrace, deck or patio is less than 0.8 metres above ground level at the boundary. | N/A  Complies  Does not comply Variation required How is the dwelling designed to meet the requirements of the standard? Describe how habitable room windows have been treated to minimise overlooking. |
| Clause 54.05-1 Daylight to new windows objective To allow adequate daylight into new habitable room windows. | Standard A16 A window in a habitable room should be located to face: - An outdoor space clear to the sky or a light court with a minimum area of 3 square metres and minimum dimension of 1 metre clear to the sky, not including land on an abutting lot, or - A verandah provided it is open for at least one third of its perimeter, or - A carport provided it has two or more open sides and is open for at least one third of its perimeter. | N/A  Complies  Does not comply Variation required Describe how adequate daylight is achieved for new habitable room windows. |
| Clause 54.05-2 Private open space objective To provide adequate private open space for the reasonable recreation and service needs of residents. | Standard A17 A dwelling should have private open space of an area and dimensions specified in the schedule to the zone. If no area or dimensions are specified in the schedule to the zone, a dwelling should have private open space consisting of an area of 80 square metres or 20 per cent of the area of the lot, whichever is the lesser, but not less than 40 square metres. At least one part of the private open space should consist of secluded private open space with a minimum area of 25 square metres and a minimum dimension of 3 metres at the side or rear of the dwelling with convenient access from a living | N/A  Complies  Does not comply Variation required Describe the minimum requirements and whether the dwelling meets the standard. |
| Clause 54.05-3 Solar Access to Open Space To allow solar access into the secluded private open space of new dwelling. | Standard A18 The private open space should be located on the north side of the dwelling, if appropriate. The southern boundary of secluded private open space should be set back from any wall on the north of the space at least (2+0.9h) metres, where ‘h’ is the height of the wall. | N/A  Complies  Does not comply Variation required How does the proposed secluded private open space receive adequate solar access and comply with the setback requirement of the standard? |
| Clause 54.06-1 Design Detail To encourage design detail that respects the existing or preferred neighbourhood character. | Standard A19 The design of buildings, including: - Façade articulation and detailing, - Window and door proportions, - Roof form, and - Verandahs, eaves and parapets, should respect the existing or preferred neighbourhood character. Garages and carports should be visually compatible with the development and the existing or preferred neighbourhood character. | N/A  Complies  Does not comply Variation required Describe how the proposed dwelling respects the preferred neighbourhood. Comment on the appropriate use and design of façade articulation & detailing, window and door proportions, roof form and verandahs, eaves and parapets (Note: Councils Urban Design Guidelines) Is the garage/carport considered to be visually compatible with the development and neighbourhood character? |
| Clause 54.06-2 Front Fences To encourage front fence design that respects the existing or preferred neighbourhood character. | Standard A20 The design of front fences should complement the design of the dwelling and any front fences on adjoining properties. A front fence within 3 metres of a street should not exceed: - Streets in a Road Zone, Category 1: 2 metres. - Other streets: 1.5 metres. Residential Growth Zone 1 - 4 1.2 metres General Residential Zone 1, 2 & 4 1.2 metres Neighbourhood Residential Zone 2 & 3 1.2 metres | N/A  Complies  Does not comply Variation required How does the proposed front fence respect the existing or preferred neighbourhood character? Comment on the proposed height and whether it meets the height requirements of the standard. |

Clause 52.06 – Car Parking Response

- Comment on whether the required number of car spaces is met.

Things to consider: Does one car space need to be undercover? Is there a tandem car space proposed? (If a tandem space is proposed an additional 500 mm in length must be provided between each space).

- Comment on the design standards under Clause 52.06-8 ensuring the standards are met.

Things to consider: Do the garage/carport dimensions meet with this clause? Does the accessway have an internal radius of at least 4 metres at changes of direction or is at least 4.2 metres wide? Does the accessway need to be designed so that cars exit in a forward direction? (consider whether the accessway serves 4 or more spaces or connect to a road in a Road Zone).

*For applications to reduce car parking requirement a response to Clause 52.06-6 must be provided.